ASIC Design Flow: Func. Specs., HDL, Synthesis, Floorplanning, place & route, verify in circuit.

FPGA: Func. Specs., HDL, Synthesis, place & route, download and verify.

A Verilog simulator consists of a compiler, user interface, and event-based scheduler.

Design Block, Stimulus Block

Abstractions Levels: Algorithmic, data flow, gate, switch, RTL.

signed 8 bit: -8sd104. Constant integers are signed by default. If width is specified, it’s not signed

~| Nor, ^ XOR, ^~ Xnor. casex —> treats all x and z as don’t cares (don’t check them). casez —> treats all z as don’t cares.

Sequential block —> begin end, parallel block —> fork join. Named Blocks

Two Types of UDP: Combinational and sequential.

UDP has exactly one output. min of 9 in for seq UDP and 10 in for comb UDP. Events: Named, Regular, Event OR Control, Level-Sensitive Control. wait() is used for level-sensitive. Stimulus vectors  – provide inputs to the model under test. Reference vectors –

provide expected output from the model

under test...used to check (either manually or automatically) that the model is performing as expected. `timescale <ref time units>/<time precision>.100ns/1ns means sim. time tick is 100 ns  and sim. time is rounded of to 1 ns increments. I/O: Open up to 3 files at most. $readmem<b/h>(file_name, mem_name, start_address, end_address). $random(<seed>). Seed is integer, time, or reg. returns a 32-bit signed integer. {$random()} to generate positives. `ifndef <flag> - compile code between `ifndef and the next `endif only when <flag> is not defined. `ifdef <flag> - compile code between `ifdef and the next `endif only when <flag> is defined. `else – optional.  Specifies path if `ifndef or `ifdef is not taken.  Each `ifndef or `ifdef can only have one `else associated with it. `elsif – optional.  Specifies conditional path if `ifndef or `ifdef is not taken.  Each `ifndef or `ifdef any number of `elsif associated with it. 3 kinds of events: Regular, Non-blocking, Monitor events. Races: Read-write, write-write, always-initial. Synthesis: process of converting design expressed in RTL into a netlist of gates. delay chain: two or more string consecutive nodes with a single fan-in and fan-out to add delay to a path. Use clock switch-ova instead of mux (logic). don’t use casex and use casez cautiously when synthesizing. Design Steps: structure data path: func. units, identify control points: status and control signals needed, determine control strategy: FSM, decoders, etc, determine reset strategy: what gets initialized, and structure your code. Divide and remainder synthesizable of power of 2. constant shifts —> wires only. variable shifts —> multiplexing logic. latches —> transparency problem. ROM —> registers in group must have equally spaced taps that are at least 3 registers apart. Seq UDP types —> Level-sens., Edge-sens. Reg Retiming: regs must have same clk, ff may not have both async. set and reset, must be at least 2 levels. Retiming can’t change latency. Soft cores are slower and simpler than hardcores. JTAG —> Joint Test Action Group. TDI, TDO, TCK, TMS, TRST.